Task 4: Mini Project — Smart Industrial Automation Grid

Robotics & Automation Internship — Progree

# 1. Executive Summary & Grid Architecture

This mini-project presents the end-to-end design and control architecture of an automated Industrial Smart Conveyor Sorting Grid. The system integrates ultrasonic height sensing, inductive metal proximity detection, and a high-speed motorized servo sorting diverter gate, coupled with an edge telemetry logging interface.

Controller: Programmable Logic Controller (PLC) / Microcontroller (Arduino Uno/Mega)

Sensors: HC-SR04 Ultrasonic Sensor, Inductive Proximity Sensor, TCS3200 Color Sensor

Actuators: 12V High-Torque DC Conveyor Motor (PWM modulated), MG996R Sortation Servo Gate

Edge Telemetry: JSON serial packet streaming to Master Station Logger

# 2. Production Line Assembly Workflow

Step 1 (Ingestion): Raw components enter on the motorized conveyor belt running at nominal 140 PWM speed.

Step 2 (Dimension Check): Ultrasonic sensor measures distance to determine component presence and height.

Step 3 (Material/Color Classification): Proximity sensor tests for ferrous metal vs. plastic packaging.

Step 4 (Automated Sortation): Servo gate actuates (60° deflection) to route metal items into Bin A while allowing plastic items to proceed to Line B.

Step 5 (Failsafe Jam Detection): If distance < 2.0 cm or sensor blocked for > 2 seconds, conveyor automatically halts (PWM=0) and triggers emergency red alert beacon.

# 3. Embedded Control & Failsafe Code

Complete control sequencing logic implemented with failsafes:

// Progree Robotics & Automation - Task 4 // Smart Conveyor PLC Logic & Edge Telemetry Controller #include <Servo.h> const int TRIG_PIN = 7; const int ECHO_PIN = 8; const int PROXIMITY_PIN = 2; const int CONVEYOR_MOTOR_PWM = 9; const int SERVO_GATE_PIN = 10; const int LED_STATUS_OK = 11; const int LED_FAILSAFE = 12; Servo sortGate; void setup() { pinMode(TRIG_PIN, OUTPUT); pinMode(ECHO_PIN, INPUT); pinMode(PROXIMITY_PIN, INPUT_PULLUP); pinMode(CONVEYOR_MOTOR_PWM, OUTPUT); pinMode(LED_STATUS_OK, OUTPUT); pinMode(LED_FAILSAFE, OUTPUT); sortGate.attach(SERVO_GATE_PIN); sortGate.write(0); Serial.begin(115200); } long measureDistance() { digitalWrite(TRIG_PIN, LOW); delayMicroseconds(2); digitalWrite(TRIG_PIN, HIGH); delayMicroseconds(10); digitalWrite(TRIG_PIN, LOW); long duration = pulseIn(ECHO_PIN, HIGH, 25000); return (duration == 0) ? 999 : (duration * 0.034 / 2); } void loop() { long distance = measureDistance(); bool objectDetected = (distance > 2 && distance < 15); bool isMetal = (digitalRead(PROXIMITY_PIN) == LOW); if (objectDetected) { digitalWrite(LED_STATUS_OK, HIGH); digitalWrite(LED_FAILSAFE, LOW); analogWrite(CONVEYOR_MOTOR_PWM, 180); if (isMetal) { sortGate.write(60); Serial.println("{"part": "METAL_PART", "action": "DIVERTER_A"}"); } else { sortGate.write(0); Serial.println("{"part": "PLASTIC_PART", "action": "STANDARD_LINE"}"); } delay(1500); sortGate.write(0); } else if (distance < 2) { analogWrite(CONVEYOR_MOTOR_PWM, 0); digitalWrite(LED_STATUS_OK, LOW); digitalWrite(LED_FAILSAFE, HIGH); Serial.println("{"part": "JAM_ERROR", "action": "EMERGENCY_STOP"}"); delay(1000); } else { analogWrite(CONVEYOR_MOTOR_PWM, 140); } delay(100); }

# 4. Edge Communication & Master Telemetry Logger

Telemetry packets are serialized in real-time JSON format and archived for SCADA/MES analytics:

# Progree Robotics & Automation — Task 4 # Edge IoT Telemetry Receiver & CSV Logger import json import csv import datetime def simulate_telemetry_stream(): sample_packets = [ {"timestamp": datetime.datetime.now().isoformat(), "part": "PLASTIC_PART", "distance_cm": 8.4, "action": "STANDARD_LINE", "conveyor_rpm": 120}, {"timestamp": datetime.datetime.now().isoformat(), "part": "METAL_PART", "distance_cm": 6.2, "action": "DIVERTER_A", "conveyor_rpm": 120}, {"timestamp": datetime.datetime.now().isoformat(), "part": "JAM_ERROR", "distance_cm": 1.2, "action": "EMERGENCY_STOP", "conveyor_rpm": 0} ] csv_file = "telemetry_production_log.csv" with open(csv_file, "w", newline="") as f: writer = csv.DictWriter(f, fieldnames=["timestamp", "part", "distance_cm", "action", "conveyor_rpm"]) writer.writeheader() for p in sample_packets: writer.writerow(p) print(f"Logged {len(sample_packets)} telemetry packets into {csv_file}") if __name__ == "__main__": simulate_telemetry_stream()